Point level detection for bins and chutes

A diaphragm level switch is a type of pressure switch used to detect Low or Hi Levels in silos/bins containing dry, non-hazardous free flowing materials. Diaphragm switch works by activating a sensitive micro-switch when material reaches the level of the switch in the Silo / bin. Output	switch can be used to start or stop a process or alert to a high, medium, or low-level

detection.

# Features:

- Point level detection in bins and silos

Rugged construction, simple design

- Durable Cast Aluminum housing

- Corrosion resistant Paint

- Silicone diaphragm material

- external mounting.

- Monitoring levels of dry, free flowing materials

- Eliminate overflow or maintain a level specific of material

- Detect overfilling/plugging of chutes or conveyors

- Material density from 20 lb./cu. ft. • Installation in ordinary locations

# Applications:

- Dry fertilizer and granular and chemicals, • Powders, bentonite, or potash

- resin pellets, regrinds, Plastic and rubber beads

- Wood pellets, chips, or sawdust

- Nut crops such as peanuts, sunflower, or sesame seeds

- Foodstuffs such as rice, wheat, oats, or sugar beets.

# SPECIFICATIONS:

Ruber Diaphragm Switch Model No: RDLS-1400S

Normal sensitivity: 200 gm (Rubber) / 400 gm (SS) Diaphragm Membrane material: Nitrile Rubber or, SS Ambient temperature: 60° C

Electrical contacts: Micro switch, 15 Amp,230 VAC, Potential free Cable Gland: PG 13 / PG 16 Main body:

Cast Aluminum

Switching Housing: Cast Aluminum for Rubber type and Polymer for SS type

Protection for switching housing: IP 67

# Drawings:

Manufactured By:

## Levtron Instruments Pvt Ltd.

Office No 2, 1st floor, plot no 6, Pandhari Industrial estate, Shivane. Pune 411023, Contact: 09579430012 / 9730042725 E-mail: sales@levtron.in Website : www.Levtron.in