Description:

Side mounted level switch used to control liquid level in vessels containing corrosive or non-corrosive liquids. It is used to pump control, Alarm, valve control or level indication.

Principle

The Float lever Carries a permanent magnet which is repelled by similar magnet mounted in the switch housing. the magnet having the same pole directed towards each other are separated by a non-magnetic diaphragm. The gland less construction offer excellent sealing.

Construction

It consists of 3 main parts:

- Housing with Micro Switch

- Mounting Flange

- Float Assembly

# Features

- Easy to install

- Reliable

- Available with different flange size

- Available in different MOC

# Application

Water, Oil, Fuels, Chemical, Solvents, Syrup etc.,

Technical Specification

Switch Type            :          Micro Switch , SPDT

Switch Rating          :           240 V AC, 5A

Enclosure                 :           Weather Proof , Flame Proof

MOC                           :           Cast Aluminum ,SS ,PP

Flange                       :           SS 304 Also Available In ANSI ,BIS,DIN                        Standard Flange

Moc                           :           SS304, SS316 ,PP

Float                         :           Dia 50,60,75 , Non Standard

Max Temperature:         150 C (Hi Temperature on Request

Max Pressure         :          20KG /CM( Higher Pressure On Request)

Manufactured By:

Levtron Instruments Pvt Ltd.

Office No 2, 1st floor, plot no 6, Pandhari Industrial estate, Shivane. Pune 411023,

Contact: 09579430012 / 9730042725 E-mail: sales@levtron.in ;

Website: www.Levtron.in