Conductivity level switches are reliable, maintenance-free point level controllers for conductive liquids (≥20 µS/cm), utilizing stainless steel probes to detect liquid presence without any moving parts. Ideal for pump control and alarms in tanks/sumps, they operate at low AC voltage, with options for rigid/flexible probes, Standard temperatures

up to 120°C, and pressure up to 10 bar, and optionally up to 250°C and pressure up to 20 bar.

Conductivity level switches use a low AC voltage between electrodes to detect the presence of a conductive liquid and are used in applications such as water treatment, sumps, and the food industry. We are offering Various models according to customer requirements with different specifications.

Specifications:

Model: CPLS-1700L

Make: Levtron Instruments Pvt Ltd. Pune

## Type:     	    	       Remote Electronics

Supply:    	   	230 V AC/ 24V DC Optional

Output:    	    	 5 Amps, SPDT X 2 Nos Potential free Relay.

Sensitivity:     	Water, Any conductive Liquid.

Conductivity:  	 ≥20 µS/cm and More

Enclosure: 		ABS, Din rail suitable for Indoor installation

Level 1:    	                              Possible in Pump Control logic / Single Point

Level 2:    	                                Possible in Pump Control logic / 2nd Point

Ref/Ground:           	 Specify Length

| Indications: | On Controller (YELLOW, RED, GREEN) |
| --- | --- |

Process connection: 	Single Point: individual probe ½” BSP/NPT   Onwards,

2 Point Probe: 1-1/2” Screwed / Flanged,

Optionally 2” HDPE/LDPE, Thickness: 10 MM ( Specify)

Probe Length:    			25 MM Onwards  up to 20 Mtrs.  (Specify)

Level 1:    	                                	ROD/ROPE, Specify Length

Level 2:    	                               	 ROD/ROPE, Specify Length

Ref/Ground:           	 Specify Length

Counterweights:    	 SS 316    x 3    In case of Rope Type

Operating Temp:     	0 TO 120° C    / Optional 250°C

Operating Pressure: Std: Up to 10 Bar/ Optional 20 Bars

Service:  STP, Water applications, Conductive liquids.

Enclosure:			Cast Aluminium, Weatherproof

Glands:    			PG 16 X 1 Nos PVC Glands

1)  Rope Probe:

# Levtron Instruments Pvt Ltd.

# Office No 02 , 1st  Floor ,Plot No 6,  Sr. No. 78/1, Pandhari Industrial Estate, Shivane, Pune-411023

# GSTIN : 27AAFCL7569M1ZP CIN: U26517PN2024PTC229033

Mob No : 9730042725 /9579430012     Email: Sales@Levtron.in