Overview: An AC electric signal is generated at the tip of the sensing electrode. If the sensor is covered with medium,

the capacitance value @ the sense probe changes. This change is detected by the electronics and converted into a switching signal.

Any build-up is ignored to a certain degree and therefore has no influence on the measurement

# Applications:-

The MINICAP is a capacitive level sensor typically used in level detection of fine-grained and powdery bulk solids, or liquid media

Typical applications are overfilled and dry run protection. The small sensor can also be used in thin pipelines for liquid level detection.

# Features:-

Simple connections

Maximum Working Life, because wear and maintenance free Design Optimized for light, powdered bulk materials, Exact switching function independent of process condition

# Technical Details:-

Supply: - 12-35 VDC

Output signal: -Std. PNP /SPDT Relay output (Specify) Process fitting: - Thread ½”, BSP Male threading (Specify) Max pressure: - 1 to 10 Bar

Max temp-: --40 To 125 Deg C

# Electrical connection:-

- Voltage supply + V 12 V DC to 35 V DC

- Gnd / 0 V

- NO	Of Relay /PNP -NO @ 350 mA max

- Pole and	5	NC

Mechanical Drawing: