Magnetic Level Indicator: LMLI-2100L

Magnetic Level Indicator / Gauge provides clear, high-quality indication, are principally designed as an alternative to glass level gauges. Magnetic Level Gauge is available in Top and Side mounted construction.

Magnetic Level Gauge consists of three major components: Float chamber, Float and Indicator system. The visual indicator is available in two types Follower Capsule and Bi-Colour Flapper.

Magnetic Level Indicator operates on principles of magnetic field coupling, to provide fluid level information. Float chamber is typically constructed with non-magnetic pipe having process connection that matches to the vessel connections. Float size and weight is determined by the process fluid, pressure, temperature and the specific gravity of process fluid. Float contains magnets to provide 3600 magnetic flux field.

Simple non-magnetic pipe with flange allows a magnet connected to the Float move up and down inside the tube. An external Bi-Colour Flapper channel or Follower Capsules indicates the level based on magnet position.

Bi-Colour Flapper Design:

Indicator system consists of Bi-Colour Flapper equipped with magnets mounted on rail inside the housing. As the level starts rising or falling magnetic Float also travels with liquid level in non-magnetic chamber. The magnetic interaction between magnets in Float and Bi-Colour Flapper causes each Flapper to rotate 1800

Follower Capsule Design:

Indicator system consists of colour Follower Capsule which moves in glass tube inside the housing. As the level starts rising or falling, magnetic Float also travels with liquid level in non-magnetic chamber. The magnetic interaction between magnets in Float and Capsule causes Capsule to travel along with magnetic Float.

For application where high and low electrical signals are required adjustable limit switch assemblies are provided. In application where continuous output is required, Reed transmitter are supplied.

Application:

Suitable for aggressive, combustible, toxic, high pressure & high temperature liquid storage tanks. It is most widely used in Fuels, Oil, Chemical, Acids, Solvents, Syrups, Beverages, Water, dyes.

# Side Mounted Magnetic Level Indicator

| 12 | Level Switch |
| --- | --- |
| 1 1 | Drain Plug |
| 10 | Level Transmitter |
| 09 | Mounting Clamp |
| 08 | Flapper |
| 07 | Vent Plug |
| 06 | Scale |
| 05 | Float Chamber |
| 04 | Process Connection |
| 03 | Chamber Flange |
| 02 | Float Rod |
| 01 | Float |
| Sl.No. | part |

TOP Mounted magnetic Level Indicator

Detail view of Indicator

Specifications

Mounting Type Indicator:              Length (C/C Distance)

Indicator system

Housing for Indicator

MOC Of Bi-Colour Flapper

Float chamber

MOC Of Float chamber

Float

Scale

Process Connection

Vent

Drain

Maximum Temperature

Pressure ordering    information LMLI2100L-

Side Mounting-S Top mounting-T

300MM To 6000MM

Bi-Colour Flapper / Follower Capsule SS housing For SS Flapper

Aluminium Housing for Aluminium Flapper SS 304/ Aluminium

50NB No-Magnetic Standard Pipe (other size on request)

Follower capsule-Glass Tube in Aluminium/SS Channel

SS 304, SS 316, SS3161, PR PVDF, Others on Request

SS 304, SS 316, SS3161, PR PVDF, Others on Request

Flange/Thread/Tri Clover/Union In Various Sizes

½” Plugged / ½” Ball Valve

½” Plugged / ½” Ball Valve

700C     for PP, 1500C for SS, Higher Temperature on Request

2kg/cm 2For PP, 10kg/cm 2For SS, More Pressure on request

.

Levtron Instruments Pvt Ltd

Office No-02, 1St Floor, Plot No 6, Sr No.78/1, Pandhari Industrial Estate, Shivane, Pune-411023

Mob No: 9730042725 /9579430012

GSTIN : 27AAFCL7569M1ZP