Levtron Instruments Pvt Ltd.

# INTRODUCTION

The Levtron Float & Board Level Indicators are designed for an

accurate, reliable & trouble free float operated technique used in large non

-pressurized storage tanks.

# PRINCIPLE

The Float & Board Level Indicators consists of a float connected to a pointer through a rope via a set of pulleys. The pointer glides over a calibrated gauge board, positioned parallel to the tank. The float accurately follows liquid level variations in vertical direction. The vertical float movement is such that the pointer in ‘TOP POSITION’ when tank is ‘EMPTY’ and in ‘BOTTOM POSITION’ when ‘FULL’.

FILPRO

# Gauge Classification

Measuring Range	:

Float	:

Calibrated Gauge Board : Least Count	:

Maximum Temperature : Maximum Pressure   :

Float Wire Rope	:

Installation	:

Specific Gravity of Fluid : Connecting Pipe	: Local Indication

Direct	:

Follower Magnet	:

Pulley Assemblies	: (2 Nos.)

Protection Conduit

Horizontal Limb	:

Vertical Limb	: Process Connections: Gauge Board Brackets  :

Special Features

Seal Pot	:

Adjustable S/W	:

Note	:

Up to 25 meters. For more than 25 meters please consult factory. 12 “ or 15 “

6” wide x Al white powder coating with black graduations & numericals 5mm

80°C (PP) / 150°C (SS)

5 Kg/cm2

Ø1.6mm x SS 316 / SS 304 (multistrand) / Ø6mm x PP (multistrand) Grounded Level Tank / Underground Tank (sump) / Overhead Tank 0.75

Carbon Steel Powder Coated

Powder coated Red CI pointer with Nylon Rollers to provide super smooth movement along gauge board. Pointer Tip indicates ‘Level’.

Red Capsule moving inside indicator tube. White annular line on capsule indicates ‘Level’. Cadmium Plated Steel Pulley with PTFE bush and SS shaft housed in weather proof CI enclosure.

GI pipe 25NB x 400mm to 800mm (adjustable) GI pipe 25NB x 195mm & 100mm long

1” 150#

Powder coated MS

For Tanks with vapour pressure upto 600 mm WC or corrosive fumes Reed S/W - 40VA (N/O), 60VA (C/O),

Micro S/W - 5A, 230 VAC (C/O)

Custom made assemblies can be provided for specific applications

Levtron Instruments Pvt Ltd.

Float & Board Level Indicator Assembly

Float Assembly

ORDERING INFORMATION

BASIC MODEL

MATERIAL OF PROCESS CONNECTION

CS = Carbon Steel	S1 = SS 304

S2 = SS 316

PROCESS CONNECTION

F501 = ½” 150# RF F503 = ½” 300# RF F506 = ½” 600# RF F751 = 3/4” 150# RF F753 = 3/4” 300# RF F756 = 3/4” 600# RF

XXX = NON STANDARD

Flanged Connections(As per ANSI B 16.5)

F101 = 1” 150# RF

F103 = 1” 300# RF

F106 = 1” 600# RF F151 = 1½” 150# RF F153 = 1½” 300# RF F156 = 1½” 600# RF

F201 = 2” 150# RF

F203 = 2” 300# RF

F206 = 2” 600# RF

F301 = 3” 150# RF

F303 = 3” 300# RF

F306 = 3” 600# RF

MATERIAL OF FLOAT

S2 = SS 316

S3 = SS 316 L

MATERIAL OF SCALE

AL = Aluminum (Standard)

S1 = SS 304

RANGE

Please specify in mm (e.g. Write 2000 for 2000 mm)

(This indicator will be supplied from 1000 mm up to 20000 mm)

OPTIONS

AR1 = Adjustable Reed Switch x 1 AR2 = Adjustable Reed Switch x 2 AM1 = Adjustable Micro Switch x 1 AM2 = Adjustable Micro Switch x 2

Levtron Instruments Pvt Ltd.

Office No 02, 1st Floor, Plot No 6, Sr. No. 78/1, Pandhari Industrial Estate, Shivane,

Pune- 411023 Mob No: 9730042725 /9579430012

Web: www.levtron.in   Email: Sales@levtron.in